CONFIGURAÇÕES PACKET TRACER

Projeto Final — CodeSecure, Lda.

CyberSecurity nas Empresas

Todas as configurações de routers, switches e servidores

ÍNDICE

1. ISP Router

2. Firewall Principal (Router 2911)

3. Core-SW1 (Switch Layer 3)

4. Core-SW2 (Switch Layer 3)

5. SW-RH (Switch de Acesso)

6. SW-Prog (Switch de Acesso)

7. SW-Suporte (Switch de Acesso)

8. Switch-DMZ

9. Switch0 (VM’s Clientes)

10. Switch1 (Data Center)

11. Servidores — IPs Estáticos

12. DHCP — Configuração no Servidor AD-DC01

13. Serviços — DNS, Web, Mail e FTP

14. ACLs — Controlo de Tráfego entre Zonas

15. Hardening — Segurança de Equipamentos

# 1. ISP Router (2911)

Simulação da ligação à Internet

enable

configure terminal

hostname ISP

interface GigabitEthernet0/0

ip address 200.200.200.1 255.255.255.252

no shutdown

exit

ip route 200.200.200.8 255.255.255.248 200.200.200.2

ip route 200.200.200.16 255.255.255.248 200.200.200.2

end

write memory

# 2. Firewall Principal (Router 2911)

Ponto central de controlo — NAT, Routing, Separação de zonas

enable

configure terminal

hostname Firewall-Principal

! Interface para o ISP (publica)

interface GigabitEthernet0/0

description LINK-ISP

ip address 200.200.200.2 255.255.255.252

ip nat outside

no shutdown

! Interface para Core-SW1

interface GigabitEthernet0/1

description LINK-CORE-SW1

ip address 192.168.1.1 255.255.255.252

ip nat inside

no shutdown

! Interface para Core-SW2

interface GigabitEthernet0/2

description LINK-CORE-SW2

ip address 192.168.2.1 255.255.255.252

ip nat inside

no shutdown

! Rota default para o ISP

ip route 0.0.0.0 0.0.0.0 200.200.200.1

! Rotas para as redes internas via Core-SW1 (primario)

ip route 192.168.10.0 255.255.255.0 192.168.1.2

ip route 192.168.20.0 255.255.255.0 192.168.1.2

ip route 192.168.30.0 255.255.255.0 192.168.1.2

ip route 192.168.40.0 255.255.255.0 192.168.1.2

ip route 192.168.50.0 255.255.255.0 192.168.1.2

ip route 192.168.60.0 255.255.255.0 192.168.1.2

ip route 192.168.99.0 255.255.255.0 192.168.1.2

! Rotas backup via Core-SW2 (metrica maior = backup)

ip route 192.168.10.0 255.255.255.0 192.168.2.2 10

ip route 192.168.20.0 255.255.255.0 192.168.2.2 10

ip route 192.168.30.0 255.255.255.0 192.168.2.2 10

ip route 192.168.40.0 255.255.255.0 192.168.2.2 10

ip route 192.168.50.0 255.255.255.0 192.168.2.2 10

ip route 192.168.60.0 255.255.255.0 192.168.2.2 10

ip route 192.168.99.0 255.255.255.0 192.168.2.2 10

! NAT estatico - DMZ Servicos

ip nat inside source static 192.168.50.10 200.200.200.10

ip nat inside source static 192.168.50.11 200.200.200.11

ip nat inside source static 192.168.50.12 200.200.200.12

! NAT estatico - DMZ Clientes

ip nat inside source static 192.168.60.10 200.200.200.20

ip nat inside source static 192.168.60.11 200.200.200.21

! PAT para os departamentos internos

access-list 1 permit 192.168.10.0 0.0.0.255

access-list 1 permit 192.168.20.0 0.0.0.255

access-list 1 permit 192.168.30.0 0.0.0.255

access-list 1 permit 192.168.40.0 0.0.0.255

ip nat inside source list 1 interface GigabitEthernet0/0 overload

end

write memory

# 3. Core-SW1 (Switch Layer 3 — 3560)

Core switch primário — HSRP Active, Root Primary, SVIs

enable

configure terminal

hostname Core-SW1

ip routing

vlan 10

name RH

vlan 20

name Programacao

vlan 30

name Suporte

vlan 40

name DataCenter

vlan 50

name DMZ-Servicos

vlan 60

name DMZ-Clientes

vlan 99

name Gestao

exit

! Interface para o Firewall (routed port)

interface GigabitEthernet1/0/1

description LINK-FIREWALL

no switchport

ip address 192.168.1.2 255.255.255.252

no shutdown

! Interface para Core-SW2 (inter-core link)

interface GigabitEthernet1/0/23

description LINK-CORE-SW2

no switchport

ip address 192.168.3.1 255.255.255.252

no shutdown

! Trunk para SW-RH

interface GigabitEthernet1/0/2

description TRUNK-SW-RH

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 10,99

no shutdown

! Trunk para SW-Prog

interface GigabitEthernet1/0/3

description TRUNK-SW-PROG

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 20,99

no shutdown

! Trunk para SW-Suporte

interface GigabitEthernet1/0/4

description TRUNK-SW-SUPORTE

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 30,99

no shutdown

! Trunk para Core-SW2 (VLANs)

interface GigabitEthernet1/0/5

description TRUNK-CORE-SW2

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,30,40,50,60,99

no shutdown

! Trunk para Switch-DMZ

interface GigabitEthernet1/0/10

description TRUNK-SWITCH-DMZ

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 50,60,99

no shutdown

! SVIs - Gateways (HSRP Active)

interface Vlan10

description GATEWAY-RH

ip address 192.168.10.2 255.255.255.0

standby 10 ip 192.168.10.1

standby 10 priority 110

standby 10 preempt

no shutdown

interface Vlan20

description GATEWAY-PROGRAMACAO

ip address 192.168.20.2 255.255.255.0

standby 20 ip 192.168.20.1

standby 20 priority 110

standby 20 preempt

no shutdown

interface Vlan30

description GATEWAY-SUPORTE

ip address 192.168.30.2 255.255.255.0

standby 30 ip 192.168.30.1

standby 30 priority 110

standby 30 preempt

no shutdown

interface Vlan40

description GATEWAY-DATACENTER

ip address 192.168.40.2 255.255.255.0

standby 40 ip 192.168.40.1

standby 40 priority 110

standby 40 preempt

no shutdown

interface Vlan50

description GATEWAY-DMZ-SERVICOS

ip address 192.168.50.2 255.255.255.0

standby 50 ip 192.168.50.1

standby 50 priority 110

standby 50 preempt

no shutdown

interface Vlan60

description GATEWAY-DMZ-CLIENTES

ip address 192.168.60.2 255.255.255.0

standby 60 ip 192.168.60.1

standby 60 priority 110

standby 60 preempt

no shutdown

interface Vlan99

description GATEWAY-GESTAO

ip address 192.168.99.2 255.255.255.0

standby 99 ip 192.168.99.1

standby 99 priority 110

standby 99 preempt

no shutdown

! DHCP Relay

interface Vlan10

ip helper-address 192.168.40.10

interface Vlan20

ip helper-address 192.168.40.10

interface Vlan30

ip helper-address 192.168.40.10

! Rota default para o Firewall

ip route 0.0.0.0 0.0.0.0 192.168.1.1

! Spanning-tree - Root Primary

spanning-tree vlan 10,20,30,40,50,60,99 root primary

end

write memory

# 4. Core-SW2 (Switch Layer 3 — 3560)

Core switch secundário — HSRP Standby, Root Secondary

enable

configure terminal

hostname Core-SW2

ip routing

vlan 10

name RH

vlan 20

name Programacao

vlan 30

name Suporte

vlan 40

name DataCenter

vlan 50

name DMZ-Servicos

vlan 60

name DMZ-Clientes

vlan 99

name Gestao

exit

! Interface para o Firewall (routed port)

interface GigabitEthernet1/0/1

description LINK-FIREWALL

no switchport

ip address 192.168.2.2 255.255.255.252

no shutdown

! Interface para Core-SW1 (inter-core link)

interface GigabitEthernet1/0/24

description LINK-CORE-SW1

no switchport

ip address 192.168.3.2 255.255.255.252

no shutdown

! Trunk para SW-RH (redundancia)

interface GigabitEthernet1/0/2

description TRUNK-SW-RH-BACKUP

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 10,99

no shutdown

! Trunk para SW-Prog (redundancia)

interface GigabitEthernet1/0/3

description TRUNK-SW-PROG-BACKUP

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 20,99

no shutdown

! Trunk para SW-Suporte (redundancia)

interface GigabitEthernet1/0/4

description TRUNK-SW-SUPORTE-BACKUP

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 30,99

no shutdown

! Trunk para Core-SW1 (VLANs)

interface GigabitEthernet1/0/5

description TRUNK-CORE-SW1

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,30,40,50,60,99

no shutdown

! Trunk para Switch1 (Data Center)

interface GigabitEthernet1/0/6

description TRUNK-SWITCH1-DATACENTER

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 40,99

no shutdown

! Trunk para Switch-DMZ

interface GigabitEthernet1/0/10

description TRUNK-SWITCH-DMZ

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 50,60,99

no shutdown

! SVIs - Gateways (HSRP Standby)

interface Vlan10

description GATEWAY-RH-STANDBY

ip address 192.168.10.3 255.255.255.0

standby 10 ip 192.168.10.1

standby 10 priority 100

standby 10 preempt

no shutdown

interface Vlan20

description GATEWAY-PROGRAMACAO-STANDBY

ip address 192.168.20.3 255.255.255.0

standby 20 ip 192.168.20.1

standby 20 priority 100

standby 20 preempt

no shutdown

interface Vlan30

description GATEWAY-SUPORTE-STANDBY

ip address 192.168.30.3 255.255.255.0

standby 30 ip 192.168.30.1

standby 30 priority 100

standby 30 preempt

no shutdown

interface Vlan40

description GATEWAY-DATACENTER-STANDBY

ip address 192.168.40.3 255.255.255.0

standby 40 ip 192.168.40.1

standby 40 priority 100

standby 40 preempt

no shutdown

interface Vlan50

description GATEWAY-DMZ-SERVICOS-STANDBY

ip address 192.168.50.3 255.255.255.0

standby 50 ip 192.168.50.1

standby 50 priority 100

standby 50 preempt

no shutdown

interface Vlan60

description GATEWAY-DMZ-CLIENTES-STANDBY

ip address 192.168.60.3 255.255.255.0

standby 60 ip 192.168.60.1

standby 60 priority 100

standby 60 preempt

no shutdown

interface Vlan99

description GATEWAY-GESTAO-STANDBY

ip address 192.168.99.3 255.255.255.0

standby 99 ip 192.168.99.1

standby 99 priority 100

standby 99 preempt

no shutdown

! DHCP Relay

interface Vlan10

ip helper-address 192.168.40.10

interface Vlan20

ip helper-address 192.168.40.10

interface Vlan30

ip helper-address 192.168.40.10

! Rotas

ip route 0.0.0.0 0.0.0.0 192.168.2.1

ip route 0.0.0.0 0.0.0.0 192.168.3.1 10

! Spanning-tree - Root Secondary

spanning-tree vlan 10,20,30,40,50,60,99 root secondary

end

write memory

# 5. SW-RH (Switch de Acesso — 2960)

Switch do departamento de Recursos Humanos — VLAN 10

enable

configure terminal

hostname SW-RH

vlan 10

name RH

vlan 99

name Gestao

exit

! Trunk para Core-SW1

interface FastEthernet0/1

description TRUNK-CORE-SW1

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 10,99

no shutdown

! Trunk para Core-SW2 (redundancia)

interface FastEthernet0/4

description TRUNK-CORE-SW2

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 10,99

no shutdown

! Portas de acesso para PCs do RH

interface range FastEthernet0/2-3

description PCS-RH

switchport mode access

switchport access vlan 10

spanning-tree portfast

no shutdown

interface Vlan99

ip address 192.168.99.11 255.255.255.0

no shutdown

ip default-gateway 192.168.99.1

end

write memory

# 6. SW-Prog (Switch de Acesso — 2960)

Switch do departamento de Programação — VLAN 20

enable

configure terminal

hostname SW-Prog

vlan 20

name Programacao

vlan 99

name Gestao

exit

! Trunk para Core-SW1

interface FastEthernet0/1

description TRUNK-CORE-SW1

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 20,99

no shutdown

! Trunk para Core-SW2 (redundancia)

interface FastEthernet0/4

description TRUNK-CORE-SW2

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 20,99

no shutdown

! Portas de acesso para PCs

interface range FastEthernet0/2-3

description PCS-PROGRAMACAO

switchport mode access

switchport access vlan 20

spanning-tree portfast

no shutdown

interface Vlan99

ip address 192.168.99.12 255.255.255.0

no shutdown

ip default-gateway 192.168.99.1

end

write memory

# 7. SW-Suporte (Switch de Acesso — 2960)

Switch do departamento de Suporte Técnico — VLAN 30

enable

configure terminal

hostname SW-Suporte

vlan 30

name Suporte

vlan 99

name Gestao

exit

! Trunk para Core-SW1

interface FastEthernet0/1

description TRUNK-CORE-SW1

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 30,99

no shutdown

! Trunk para Core-SW2 (redundancia)

interface FastEthernet0/4

description TRUNK-CORE-SW2

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 30,99

no shutdown

! Portas de acesso para PCs

interface range FastEthernet0/2-3

description PCS-SUPORTE

switchport mode access

switchport access vlan 30

spanning-tree portfast

no shutdown

interface Vlan99

ip address 192.168.99.13 255.255.255.0

no shutdown

ip default-gateway 192.168.99.1

end

write memory

# 8. Switch-DMZ (2960)

Switch da zona DMZ de Serviços — VLAN 50

enable

configure terminal

hostname Switch-DMZ

vlan 50

name DMZ-Servicos

vlan 99

name Gestao

exit

! Trunk para Core-SW1

interface GigabitEthernet0/1

description TRUNK-CORE-SW1

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 50,99

no shutdown

! Trunk para Core-SW2 (redundancia)

interface GigabitEthernet0/2

description TRUNK-CORE-SW2

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 50,99

no shutdown

! Servidores DMZ

interface FastEthernet0/2

description WEB-SERVER-DMZ

switchport mode access

switchport access vlan 50

spanning-tree portfast

no shutdown

interface FastEthernet0/3

description MAIL-GATEWAY

switchport mode access

switchport access vlan 50

spanning-tree portfast

no shutdown

interface FastEthernet0/4

description DNS-PUBLICO

switchport mode access

switchport access vlan 50

spanning-tree portfast

no shutdown

interface Vlan99

ip address 192.168.99.14 255.255.255.0

no shutdown

ip default-gateway 192.168.99.1

end

write memory

# 9. Switch0 (2960 — VM’s Clientes)

Switch da zona DMZ de Clientes — VLAN 60

enable

configure terminal

hostname Switch0

vlan 60

name DMZ-Clientes

vlan 99

name Gestao

exit

! Trunk para Core-SW2

interface GigabitEthernet0/1

description TRUNK-CORE-SW2

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 60,99

no shutdown

! Trunk para Core-SW1 (redundancia)

interface GigabitEthernet0/2

description TRUNK-CORE-SW1

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 60,99

no shutdown

! Servidores Clientes

interface FastEthernet0/2

description 14-VMS-REDHAT

switchport mode access

switchport access vlan 60

spanning-tree portfast

no shutdown

interface FastEthernet0/3

description 120-WEB-SERVERS-CLIENTES

switchport mode access

switchport access vlan 60

spanning-tree portfast

no shutdown

interface Vlan99

ip address 192.168.99.15 255.255.255.0

no shutdown

ip default-gateway 192.168.99.1

end

write memory

# 10. Switch1 (2960 — Data Center)

Switch do Data Center — VLAN 40

enable

configure terminal

hostname Switch1

vlan 40

name DataCenter

vlan 99

name Gestao

exit

! Trunk para Core-SW2

interface GigabitEthernet0/1

description TRUNK-CORE-SW2

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 40,99

no shutdown

! Trunk para Core-SW1 (redundancia)

interface GigabitEthernet0/2

description TRUNK-CORE-SW1

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 40,99

no shutdown

! Servidores Data Center

interface FastEthernet0/1

description AD-DC01-DNS-DHCP

switchport mode access

switchport access vlan 40

spanning-tree portfast

no shutdown

interface FastEthernet0/2

description SIEM-WAZUH

switchport mode access

switchport access vlan 40

spanning-tree portfast

no shutdown

interface FastEthernet0/3

description PARTILHA-FICHEIROS

switchport mode access

switchport access vlan 40

spanning-tree portfast

no shutdown

interface FastEthernet0/4

description LOG-ELK-STACK

switchport mode access

switchport access vlan 40

spanning-tree portfast

no shutdown

interface Vlan99

ip address 192.168.99.16 255.255.255.0

no shutdown

ip default-gateway 192.168.99.1

end

write memory

# 11. Servidores — IPs Estáticos

Configurar em cada servidor: Desktop → IP Configuration

## Data Center — VLAN 40

| Servidor | IP Address | Subnet Mask | Gateway | DNS |
| --- | --- | --- | --- | --- |
| AD-DC01 (AD+DNS+DHCP) | 192.168.40.10 | 255.255.255.0 | 192.168.40.1 | 127.0.0.1 |
| SIEM-Wazuh | 192.168.40.11 | 255.255.255.0 | 192.168.40.1 | 192.168.40.10 |
| LOG-ELK Stack | 192.168.40.12 | 255.255.255.0 | 192.168.40.1 | 192.168.40.10 |
| Partilha de Ficheiros | 192.168.40.13 | 255.255.255.0 | 192.168.40.1 | 192.168.40.10 |

## DMZ Serviços — VLAN 50

| Servidor | IP Address | Subnet Mask | Gateway | DNS |
| --- | --- | --- | --- | --- |
| Web-Server-DMZ | 192.168.50.10 | 255.255.255.0 | 192.168.50.1 | 192.168.50.11 |
| DNS-Público | 192.168.50.11 | 255.255.255.0 | 192.168.50.1 | 127.0.0.1 |
| Mail-Gateway | 192.168.50.12 | 255.255.255.0 | 192.168.50.1 | 192.168.50.11 |

## DMZ Clientes — VLAN 60

| Servidor | IP Address | Subnet Mask | Gateway | DNS |
| --- | --- | --- | --- | --- |
| 120 Web Servers Clientes | 192.168.60.10 | 255.255.255.0 | 192.168.60.1 | 192.168.50.11 |
| 14 VMs RedHat | 192.168.60.11 | 255.255.255.0 | 192.168.60.1 | 192.168.50.11 |

## Laptops (Departamentos)

Todos os laptops devem estar configurados em modo DHCP (Desktop → IP Configuration → DHCP). Receberão IP automaticamente após configuração do servidor DHCP.

# 12. DHCP — Configuração no Servidor AD-DC01

Configurar no servidor AD-DC01: Services → DHCP → Ligar serviço (ON)

| Pool Name | Default GW | DNS Server | Start IP | Subnet Mask | Max Users |
| --- | --- | --- | --- | --- | --- |
| VLAN10-RH | 192.168.10.1 | 192.168.40.10 | 192.168.10.100 | 255.255.255.0 | 101 |
| VLAN20-PROG | 192.168.20.1 | 192.168.40.10 | 192.168.20.100 | 255.255.255.0 | 101 |
| VLAN30-SUPORTE | 192.168.30.1 | 192.168.40.10 | 192.168.30.100 | 255.255.255.0 | 101 |

## Passos de Configuração

1. Abrir o servidor AD-DC01 → Services → DHCP

2. Certificar que o serviço está ON

3. Para cada pool: preencher os campos conforme a tabela acima e clicar Add

4. O pool default (serverPool) pode ser deixado ou removido

## DHCP Relay (já configurado nos Core Switches)

Os Core Switches (SW1 e SW2) já têm o comando ip helper-address 192.168.40.10 nas SVIs das VLANs 10, 20 e 30, garantindo que os pedidos DHCP dos laptops são encaminhados para o servidor AD-DC01.

## Resultado Esperado

| Departamento | Gama de IPs | Gateway Recebido |
| --- | --- | --- |
| RH (VLAN 10) | 192.168.10.100 - 192.168.10.200 | 192.168.10.1 |
| Programação (VLAN 20) | 192.168.20.100 - 192.168.20.200 | 192.168.20.1 |
| Suporte (VLAN 30) | 192.168.30.100 - 192.168.30.200 | 192.168.30.1 |

# 13. Serviços — DNS, Web, Mail e FTP

## 13.1 DNS Público (192.168.50.11)

Services → DNS → ON. Adicionar os seguintes registos:

| Name | Type | Address |
| --- | --- | --- |
| codesecure.pt | A Record | 192.168.50.10 |
| www.codesecure.pt | A Record | 192.168.50.10 |
| mail.codesecure.pt | A Record | 192.168.50.12 |
| dns.codesecure.pt | A Record | 192.168.50.11 |
| clientes.codesecure.pt | A Record | 192.168.60.10 |

## 13.2 DNS Interno — AD-DC01 (192.168.40.10)

Services → DNS → ON. Adicionar os seguintes registos:

| Name | Type | Address |
| --- | --- | --- |
| dc01.codesecure.local | A Record | 192.168.40.10 |
| wazuh.codesecure.local | A Record | 192.168.40.11 |
| elk.codesecure.local | A Record | 192.168.40.12 |
| files.codesecure.local | A Record | 192.168.40.13 |
| codesecure.pt | A Record | 192.168.50.10 |
| www.codesecure.pt | A Record | 192.168.50.10 |

## 13.3 Web Server DMZ (192.168.50.10)

Services → HTTP → HTTP e HTTPS ON. Editar index.html:

<html>

<head><title>CodeSecure, Lda.</title></head>

<body>

<h1>Bem-vindo a CodeSecure, Lda.</h1>

<p>Desenvolvimento de Software e Alojamento Web</p>

<p>120 sites alojados | 14 VMs de clientes</p>

</body>

</html>

## 13.4 Web Servers Clientes (192.168.60.10)

Services → HTTP → HTTP e HTTPS ON. Editar index.html:

<html>

<head><title>Alojamento Clientes - CodeSecure</title></head>

<body>

<h1>Plataforma de Alojamento de Clientes</h1>

<p>120 websites alojados na infraestrutura CodeSecure</p>

</body>

</html>

## 13.5 Mail Gateway (192.168.50.12)

Services → EMAIL → SMTP e POP3 ON. Domain: codesecure.pt

| Username | Password |
| --- | --- |
| admin | admin123 |
| rh | rh123 |
| suporte | suporte123 |

## 13.6 Servidor de Ficheiros / FTP (192.168.40.13)

Services → FTP → ON. Criar utilizador com todas as permissões:

Username: admin | Password: admin123 | Permissões: Write, Read, Delete, Rename, List

# 14. ACLs — Controlo de Tráfego entre Zonas

Políticas de segurança implementadas no Firewall Principal e Core Switches

## 14.1 ACLs no Firewall Principal

enable

configure terminal

! ============================================

! ACL PARA TRAFEGO VINDO DA INTERNET (Gig0/0)

! ============================================

ip access-list extended INTERNET-IN

permit tcp any host 192.168.50.10 eq 80

permit tcp any host 192.168.50.10 eq 443

permit udp any host 192.168.50.11 eq 53

permit tcp any host 192.168.50.11 eq 53

permit tcp any host 192.168.50.12 eq 25

permit tcp any host 192.168.50.12 eq 587

permit tcp any host 192.168.50.12 eq 993

permit tcp any host 192.168.60.10 eq 80

permit tcp any host 192.168.60.10 eq 443

permit tcp any any established

permit icmp any any

deny ip any 192.168.10.0 0.0.0.255

deny ip any 192.168.20.0 0.0.0.255

deny ip any 192.168.30.0 0.0.0.255

deny ip any 192.168.40.0 0.0.0.255

deny ip any 192.168.99.0 0.0.0.255

deny ip any any

! ============================================

! ACL PARA TRAFEGO DA REDE INTERNA (Gig0/1)

! ============================================

ip access-list extended INTERNA-IN

permit ip 192.168.10.0 0.0.0.255 192.168.40.0 0.0.0.255

deny ip 192.168.10.0 0.0.0.255 192.168.50.0 0.0.0.255

deny ip 192.168.10.0 0.0.0.255 192.168.60.0 0.0.0.255

permit ip 192.168.20.0 0.0.0.255 192.168.40.0 0.0.0.255

permit ip 192.168.20.0 0.0.0.255 192.168.50.0 0.0.0.255

permit ip 192.168.20.0 0.0.0.255 192.168.60.0 0.0.0.255

permit ip 192.168.30.0 0.0.0.255 192.168.40.0 0.0.0.255

permit ip 192.168.30.0 0.0.0.255 192.168.50.0 0.0.0.255

permit ip 192.168.30.0 0.0.0.255 192.168.60.0 0.0.0.255

permit ip 192.168.40.0 0.0.0.255 any

permit ip 192.168.99.0 0.0.0.255 any

permit ip 192.168.10.0 0.0.0.255 any

permit ip 192.168.20.0 0.0.0.255 any

permit ip 192.168.30.0 0.0.0.255 any

deny ip any any

! ============================================

! ACL PARA TRAFEGO DA DMZ (Gig0/2)

! ============================================

ip access-list extended DMZ-IN

permit tcp 192.168.50.0 0.0.0.255 host 192.168.40.11 eq 1514

permit tcp 192.168.50.0 0.0.0.255 host 192.168.40.11 eq 1515

permit tcp 192.168.50.0 0.0.0.255 host 192.168.40.12 eq 5044

permit tcp 192.168.50.0 0.0.0.255 host 192.168.40.12 eq 9200

permit udp 192.168.50.0 0.0.0.255 host 192.168.40.12 eq 5514

permit tcp 192.168.60.0 0.0.0.255 host 192.168.40.11 eq 1514

permit tcp 192.168.60.0 0.0.0.255 host 192.168.40.11 eq 1515

permit tcp 192.168.60.0 0.0.0.255 host 192.168.40.12 eq 5044

permit tcp 192.168.60.0 0.0.0.255 host 192.168.40.12 eq 9200

permit udp 192.168.60.0 0.0.0.255 host 192.168.40.12 eq 5514

permit udp 192.168.50.0 0.0.0.255 host 192.168.40.10 eq 53

permit udp 192.168.60.0 0.0.0.255 host 192.168.40.10 eq 53

permit ip 192.168.50.0 0.0.0.255 any

permit ip 192.168.60.0 0.0.0.255 any

deny ip 192.168.50.0 0.0.0.255 192.168.10.0 0.0.0.255

deny ip 192.168.50.0 0.0.0.255 192.168.20.0 0.0.0.255

deny ip 192.168.50.0 0.0.0.255 192.168.30.0 0.0.0.255

deny ip 192.168.60.0 0.0.0.255 192.168.10.0 0.0.0.255

deny ip 192.168.60.0 0.0.0.255 192.168.20.0 0.0.0.255

deny ip 192.168.60.0 0.0.0.255 192.168.30.0 0.0.0.255

deny ip any any

! APLICAR ACLs NAS INTERFACES

interface GigabitEthernet0/0

ip access-group INTERNET-IN in

interface GigabitEthernet0/1

ip access-group INTERNA-IN in

interface GigabitEthernet0/2

ip access-group DMZ-IN in

end

write memory

## 14.2 ACL nos Core Switches (Bloquear RH da DMZ)

Aplicar em AMBOS os Core Switches (SW1 e SW2):

enable

configure terminal

ip access-list extended BLOQUEAR-RH-DMZ

deny ip 192.168.10.0 0.0.0.255 192.168.50.0 0.0.0.255

deny ip 192.168.10.0 0.0.0.255 192.168.60.0 0.0.0.255

permit ip any any

interface Vlan10

ip access-group BLOQUEAR-RH-DMZ in

end

write memory

## 14.3 Resumo das Políticas de Segurança

| Origem | Destino | Resultado | Justificação |
| --- | --- | --- | --- |
| Internet | DMZ (HTTP/DNS/Mail) | PERMITIDO | Serviços públicos |
| Internet | Rede Interna | BLOQUEADO | Proteção interna |
| RH (VLAN 10) | DMZ | BLOQUEADO | Sem necessidade |
| RH (VLAN 10) | Data Center | PERMITIDO | Acesso a serviços |
| Programação (VLAN 20) | DMZ | PERMITIDO | Deploy e gestão |
| Suporte (VLAN 30) | DMZ | PERMITIDO | Suporte técnico |
| Internos | Internet | PERMITIDO | Navegação via PAT |
| DMZ | DC (Logs) | PERMITIDO | Wazuh/ELK logs |
| DMZ | Rede Interna | BLOQUEADO | Isolar DMZ |

# 15. Hardening — Segurança de Equipamentos

Configuração de segurança aplicada a TODOS os equipamentos de rede

## 15.1 Configuração Base (todos os equipamentos)

enable secret cesae123

username admin privilege 15 secret cesae123

line console 0

login local

exec-timeout 5 0

logging synchronous

line vty 0 4

login local

transport input ssh

exec-timeout 5 0

ip domain-name codesecure.pt

crypto key generate rsa general-keys modulus 2048

service password-encryption

banner motd #AVISO: Acesso restrito. Atividade monitorizada.#

Nota: Nos switches (line vty 0 15) em vez de (line vty 0 4).

## 15.2 Port Security (Switches de Acesso)

Aplicado nos switches SW-RH, SW-Prog e SW-Suporte nas portas de acesso:

interface range FastEthernet0/2-3

switchport port-security

switchport port-security maximum 2

switchport port-security violation shutdown

switchport port-security mac-address sticky

## 15.3 Desativar Portas Não Utilizadas

Exemplo para switches de acesso (SW-RH, SW-Prog, SW-Suporte):

interface range FastEthernet0/5-24

shutdown

interface range GigabitEthernet0/1-2

shutdown

## 15.4 Resumo de Medidas de Hardening

| Medida | Descrição |
| --- | --- |
| SSH | Acesso remoto encriptado, Telnet desativado |
| Autenticação Local | Username + password (secret) em todos os equipamentos |
| Timeout 5 minutos | Sessões inativas desligam automaticamente |
| Banner MOTD | Aviso legal de acesso restrito e monitorização |
| Port Security | Máx. 2 MACs por porta, sticky, shutdown em violação |
| Portas Desativadas | Todas as portas não utilizadas em shutdown |
| Password Encryption | service password-encryption ativo |
| RSA 2048 bits | Chaves SSH com 2048 bits de segurança |